# Step 1: Identify neighbors of each element in the deformed state

1. Initialize neighbor_matrix

2. Partition 3D space into a fine grid

3. Assign each node to a bin within this grid

4. For each node in an element:

a. Find other nodes in the same bin

b. For each node in the same bin:

i. Calculate the distance to the original node

ii. If distance < delta (a small threshold):

- Identify the element associated with the neighboring node

- Add this element to the neighbor list of the original element

# Step 2: Conduct a first sweep to find fragment clusters

1. Calculate the centroid position of all elements

2. Partition 3D space into a coarser grid

3. Place each element in a bin based on centroid position

4. For each bin:

a. Create a NeighborList_inBin matrix with neighbor element IDs

b. Extract clusters using a snowball procedure:

i. Start with the first row as the initial cluster

ii. Loop through each row, combining elements if there's an overlap

iii. Continue until no additional overlap is found

iv. Store the identified clusters

# Step 3: Combine clusters from the first sweep to form final global clusters

1. Combine all first sweep clusters into a single array

2. Conduct the snowball procedure on this array to extract global clusters

3. Store the global clusters, each represented as a set of element IDs